FORM DCA-CL/RVSM/AIR-OPS/01 – Compliance Checklist for MCAR Part-SPA, Subpart A: General Requirements and Subpart D: Operations in Airspace with Reduced Vertical Separation Minima (RVSM) to Air Operations and Chapter 11 of the MCAR Airworthiness

This form should only be completed electronically and printed and signed. Send your completed form to civil-aviation@govmu.org

FALSE REPRESENTATION STATEMENT

It is an offence to make, with intent to deceive, any false representations for the purpose of procuring the grant, issue, renewal or variation of any certificate, licence, approval, permission or other document. Persons doing so render themselves liable and subject to prosecution under the current applicable regulation. Refer to Regulation 86 of the Civil Aviation Regulations 2007.

Form DCA-CL/RVSM/AIR-OPS/01– Compliance Checklist for MCAR Part-SPA, Subpart A: General Requirements and Subpart D: Operations in Airspace with Reduced Vertical Separation Minima (RVSM) and Chapter 11 of the MCAR Airworthiness is divided in 5 sections. Section 1 must be completed with applicant/operator information as displayed on AOC, AOC application form or alternative document for other than CAT operations. Section 2 is divided in 5 columns and lists all respective Part-SPA, Subpart A: General Requirements and Subpart D: Operations in Airspace with Reduced Vertical Separation Minima (RVSM) implementing regulations and associated AMC/GM that are applicable to RVSM operations. Operator/applicant must provide a specific reference to a manual or other applicable document and may provide a brief narrative description that describes how the applicant will comply with each requirement. The operations manual system field under section 2 must be used to list all manual and documents used for documentation. Section 3 lists relevant requirements of the Chapter 11 of the MCAR Airworthiness or the MCAR PART-SPA where applicable. When answering to the questions/requirements of Section 3, the operator shall provide further details in the method of compliance/manual reference or remarks columns. Section 4 contains the compliance statement which must be signed by the accountable person after completion by the responsible person for flight operations. By signing the compliance statement, the signatory accepts management accountability for compliance with applicable requirements for RVSM operations. Section 5 is to be filled by the Authority only.

Where the applicant does not meet the rule requirement or deems it not applicable, the column must be annotated N/A, and an explanation given as to why it is not applicable. All sections in this compliance checklist indicate either a document reference or N/A, if not applicable.

The completed compliance checklist must be submitted by the applicant/operator both for initial approval and variations. Additionally, the operator should maintain an up-to-date compliance checklist to assist with on-going compliance and to support approval amendment requests. The intention of this checklist is to assist rather than instruct the applicant in an initial application or request for renewal.

| SECTION 1 – Operator/Applicant Contact Details This section must be completed with information as displayed on the AOC, AOC application form or alternative document for other than CAT operations as applicable |  |  |  |
| --- | --- | --- | --- |
| Name and mailing address: |  | Address of Principal Base of Operations |  |
| Phone: |  | Doing Business As (DBA): |  |
| E-mail: |  |  |  |
| AOC No. (If Applicable) |  | Aircraft Manufacturer: |  |
| Aircraft Type / Model |  | List of Aircraft (registration marks and S/N) requested for RVSM approval: |  |
| Completed by: Name: |  | Signature: | Date: |

| SECTION 2 – Compliance Checklist |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The information in the columns below must be completed by the operator. |  |  |  |  |  |  | Authority Use |  |
| Operations Manual System |  |  |  |  |  |  |  |  |
| Regulation |  |  | Method of Compliance Manual Reference | Remarks |  | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |  |  |
| SUBPART A: GENERAL REQUIREMENTS |  |  |  |  |  |  |  |  |
| SPA.GEN.100 Competent authority | (a) The competent authority for issuing a specific approval shall be: |  |  |  |  |  |  |  |
|  | (1) for the commercial operator, the authority in which the operator has its principal place of business; |  |  |  |  |  |  |  |
|  | (2) for the non-commercial operator, the authority of the State in which the operator has its principal place of business, is established or is residing. |  |  |  |  |  |  |  |
|  | (b) Notwithstanding point (a)(2), for the non-commercial operator using aircraft registered in a third country, the applicable requirements under this MCAR for the approval of the following operations shall not apply if those approvals are issued by a third-country State of Registry: |  |  |  |  |  |  |  |
|  | (1) performance-based navigation (PBN); |  |  |  |  |  |  |  |

| The columns below must be completed by the operator. |  |  |  |  | Authority Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Regulation |  | Method of Compliance Manual Reference | Remarks | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |
| SUBPART A: GENERAL REQUIREMENTS |  |  |  |  |  |  |
| SPA.GEN.100 Competent authority | (b) Notwithstanding point (a)(2), for the non- commercial operator using aircraft registered in a third country, the applicable requirements under this Annex for the approval of the following operations shall not apply if those approvals are issued by a third-country State of Registry: |  |  |  |  |  |
|  | (2) minimum operational performance specifications (MNPS); |  |  |  |  |  |
|  | (3) reduced vertical separation minima (RVSM) airspace; |  |  |  |  |  |
|  | (4) low visibility operations (LVO). GM1 SPA.GEN.100(a) Competent authority DETERMINING THE PLACE WHERE AN OPERATOR IS RESIDING |  |  |  |  |  |
| SPA.GEN.105 Application for a specific approval | (a) The operator applying for the initial issue of a specific approval shall provide to the Authority the documentation required in the applicable Subpart, together with the following information: |  |  |  |  |  |
|  | (1) the name, address and mailing address of the applicant; |  |  |  |  |  |
|  | (2) a description of the intended operation. AMC1 SPA.GEN.105(a) Application for a specific approval DOCUMENTATION |  |  |  |  |  |

| The columns below must be completed by the operator. |  |  |  |  | Authority Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Regulation |  | Method of Compliance Manual Reference | Remarks | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |
| SUBPART A: GENERAL REQUIREMENTS |  |  |  |  |  |  |
| SPA.GEN.105 Application for a specific approval | (b) The operator shall provide the following evidence to the competent authority: |  |  |  |  |  |
|  | (1) compliance with the requirements of the applicable Subpart; |  |  |  |  |  |
|  | (2) that the relevant elements defined in the mandatory part of the operational suitability data established in accordance with Civil Aviation Regulation are taken into account. |  |  |  |  |  |
|  | (c) The operator shall retain records relating to (a) and (b) at least for the duration of the operation requiring a specific approval, or, if applicable, in accordance with MCAR Part-ORO. |  |  |  |  |  |
| SPA.GEN.110 Privileges of an operator holding a specific approval | The scope of the activity that an operator is approved to conduct shall be documented and specified: |  |  |  |  |  |
|  | (a) for operators holding an air operator certificate (AOC) in the operations specifications to the AOC; |  |  |  |  |  |
|  | (b) for all other operators in the list of specific approvals. |  |  |  |  |  |
| SPA.GEN.115 Changes to a specific approval | When the conditions of a specific approval are affected by changes, the operator shall provide the relevant documentation to the competent authority and obtain prior approval for the operation. |  |  |  |  |  |

| The columns below must be completed by the operator. |  |  |  |  | Authority Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Regulation |  | Method of Compliance Manual Reference | Remarks | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |
| SUBPART A: GENERAL REQUIREMENTS |  |  |  |  |  |  |
| SPA.GEN.120 Continued validity of a specific approval | Specific approvals shall be issued for an unlimited duration and shall remain valid subject to the operator remaining in compliance with the requirements associated with the specific approval and taking into account the relevant elements defined in the mandatory part of the operational suitability data established in accordance with Civil Aviation Regulation. |  |  |  |  |  |

| The columns below must be completed by the operator. |  |  |  |  | Authority Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Regulation |  | Method of Compliance Manual Reference | Remarks | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |
| SUBPART D: OPERATIONS IN AIRSPACE WITH REDUCED VERTICAL SEPARATION MINIMA (RVSM) |  |  |  |  |  |  |
| SPA.RVSM.100 RVSM operations | Aeroplanes and helicopters shall only be operated in designated airspace where a reduced vertical separation minimum of 300 m (1 000 ft) applies between flight level (FL) 290 and FL 410, inclusive, if the operator has been granted an approval by the Authority to conduct such operations. |  |  |  |  |  |
| SPA.RVSM.105 RVSM operational approval | To obtain an RVSM operational approval from the Authority, the operator shall provide evidence that: |  |  |  |  |  |
|  | (a) the RVSM airworthiness approval has been obtained; |  |  |  |  |  |
|  | (b) procedures for monitoring and reporting height-keeping errors have been established; |  |  |  |  |  |
|  | (c) a training programme for the flight crew members involved in these operations has been established; |  |  |  |  |  |
|  | (d) operating procedures have been established specifying: |  |  |  |  |  |
|  | (1) the equipment to be carried, including its operating limitations and appropriate entries in the MEL; |  |  |  |  |  |
|  | (2) flight crew composition and experience requirements; |  |  |  |  |  |

| The columns below must be completed by the operator. |  |  |  |  | Authority Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Regulation |  | Method of Compliance Manual Reference | Remarks | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |
| SUBPART D: OPERATIONS IN AIRSPACE WITH REDUCED VERTICAL SEPARATION MINIMA (RVSM) |  |  |  |  |  |  |
| SPA.RVSM.105 RVSM operational approval | (d) operating procedures have been established specifying: |  |  |  |  |  |
|  | (3) flight planning; |  |  |  |  |  |
|  | (4) pre-flight procedures; |  |  |  |  |  |
|  | (5) procedures prior to RVSM airspace entry; |  |  |  |  |  |
|  | (6) in-flight procedures; |  |  |  |  |  |
|  | (7) post-flight procedures; |  |  |  |  |  |
|  | (8) incident reporting; |  |  |  |  |  |
|  | (9) specific regional operating procedures. AMC1 SPA.RVSM.105 RVSM operational approval CONTENT OF OPERATOR RVSM APPLICATION AMC2 SPA.RVSM.105 RVSM operational approval OPERATING PROCEDURES AMC3 SPA.RVSM.105 RVSM operational approval CONTINUING AIRWORTHINESS GM1 SPA.RVSM.105(d)(9) RVSM operational approval SPECIFIC REGIONAL PROCEDURES |  |  |  |  |  |

| The columns below must be completed by the operator. |  |  |  |  | Authority Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Regulation |  | Method of Compliance Manual Reference | Remarks | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |
| SUBPART D: OPERATIONS IN AIRSPACE WITH REDUCED VERTICAL SEPARATION MINIMA (RVSM) |  |  |  |  |  |  |
| SPA.RVSM.110 RVSM equipment requirements | Aircraft used for operations in RVSM airspace shall be equipped with: |  |  |  |  |  |
|  | (a) two independent altitude measurement systems; |  |  |  |  |  |
|  | (b) an altitude alerting system; |  |  |  |  |  |
|  | (c) an automatic altitude control system; |  |  |  |  |  |
|  | (d) a secondary surveillance radar (SSR) transponder with altitude reporting system that can be connected to the altitude measurement system in use for altitude control. AMC1 SPA.RVSM.110(a) RVSM equipment requirements TWO INDEPENDENT ALTITUDE MEASUREMENT SYSTEMS |  |  |  |  |  |
| SPA.RVSM.115 RVSM height-keeping errors | (a) The operator shall report recorded or communicated occurrences of height-keeping errors caused by malfunction of aircraft equipment or of operational nature, equal to or greater than: |  |  |  |  |  |
|  | (1) a total vertical error (TVE) of ± 90 m (± 300 ft); |  |  |  |  |  |
|  | (2) an altimetry system error (ASE) of ± 75 m (± 245 ft); and |  |  |  |  |  |
|  | (3) an assigned altitude deviation (AAD) of ± 90 m (± 300 ft). |  |  |  |  |  |

| The columns below must be completed by the operator. |  |  |  |  | Authority Use |  |
| --- | --- | --- | --- | --- | --- | --- |
| Regulation |  | Method of Compliance Manual Reference | Remarks | N/A | A | U |
| Reference | Requirement |  |  |  |  |  |
| MCAR PART-SPA |  |  |  |  |  |  |
| SUBPART D: OPERATIONS IN AIRSPACE WITH REDUCED VERTICAL SEPARATION MINIMA (RVSM) |  |  |  |  |  |  |
| SPA.RVSM.115 RVSM height-keeping errors | (b) Reports of such occurrences shall be sent to the competent authority within 72 hours. Reports shall include an initial analysis of causal factors and measures taken to prevent repeat occurrences. |  |  |  |  |  |
|  | (c) When height-keeping errors are recorded or received, the operator shall take immediate action to rectify the conditions that caused the errors and provide follow-up reports, if requested by the Authority. |  |  |  |  |  |

| SECTION 3 – RVSM Airworthiness Elements Compliance Checklist |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| The information in the columns below must be completed by the operator. Note: Documented Objective Evidence and/or Extracts of manuals must be provided to support answers listed below |  |  |  |  | Authority Use |  |
| Regulation |  | Method of Compliance / Manual Reference | Remarks | N/A | A | U |
| # | Requirement |  |  |  |  |  |
| MCAR PART-SPA and MCAR Airworthiness |  |  |  |  |  |  |
| (i) | Does the aircraft production (build) and/or modification status or aircraft type datasheet confirm the aircraft is RVSM compliant? Note: Include references to the manufacturer’s statement of RVSM compliance (if applicable). The statement of RVSM capability is also indicated in the Airplane Flight Manual (AFM) |  |  |  |  |  |
| (ii) | Has any repair been embodied on the aircraft which may affect RVSM compliance? Note: This could be related to static ports, skin waviness or to Altitude reporting systems |  |  |  |  |  |
| (iii) | Does the CAME include RVSM procedures to support RVSM operations and monitoring? If Yes, provide details in the next column and/or remarks column |  |  |  |  |  |
| (iv) | Does the Maintenance Programme embody all tasks and associated requirements for RVSM operations? If Yes, provide details in the next column and/or remarks column |  |  |  |  |  |
| (v) | Does the Reliability Porgramme (if applicable) embody all requirements for RVSM operations? If Yes, provide details in the next column and/or remarks column |  |  |  |  |  |
| (vi) | Does the Operator MEL embody all maintenance procedure and processes for upgrade/downgrade of RVSM due to system failures within RVSM critical systems? If Yes, provide details in the next column and/or remarks column |  |  |  |  |  |
| (vii) | Does the contracted Part 145 Maintenance Organisation procedures support RVSM upgrade/downgrade processes and control of aircraft geometric inspection techniques? If Yes, provide details in the next column and/or remarks column |  |  |  |  |  |
| (viii) | Has RVSM training for the CAMO and Contracted Part 145 Organisations staff been completed? If Yes, provide details in the next column and/or remarks column. |  |  |  |  |  |

| SECTION 4 – Compliance Statement |  |  |  |
| --- | --- | --- | --- |
| I certify that I have read the regulatory requirements referenced in sections 2 and 3 above and understand its applicability to the operations. I certify that the above compliance checklist is a true reflection of applicant/operator compliance to the applicable regulatory requirements for RVSM operations. As the Accountable person, I accept the responsibility for the compliance to all applicable requirements for RVSM operations. |  |  |  |
| Tittle/Appointment: | Name: | Signature: | Date: |

| SECTION 5 – For DCA Use Only |  |  |
| --- | --- | --- |
| The undersigned certifies the above information mentioned in Sections 1 to 4 have been verified as indicated in the last two columns (Authority Use) and the aircraft as listed in Section 1 of the checklist meets the requirements for RVSM operations from Airworthiness and Flight Operations aspects. Approval of RVSM operation is hereby [Recommended/Not Recommended]. |  |  |
| Air Inspector Name: [Recommended or Not recommended] | Signature: | Date: |
| FOI Name: [Recommended or Not recommended] | Signature: | Date: |

General Remarks to Airworthiness and Flight Operations Inspectors:

- Following review of the RVSM application from Airworthiness perspective, the Airworthiness Inspector shall return the completed Form DCA-CL/RVSM/AIR-OPS/01 compliance checklist with recommendation to the Flight Operations Inspector for approval.

- The Flight Operations Inspector shall not issue the RVSM approval until it has been recommended by the inspector of the Airworthiness Section.